# COPPERBELT UNIVERSITY

Improvement of a Motorized Gyroscope

Chapter 3: Methodology -- Extract: Section 3.1 Component Design and Section 3.2 Component Analysis

# 3.1 Component Design

This section presents the design of each mechanical and electrical component of the gyroscope, in the order they were selected: flywheel, shaft, bearings, motor and ESC, coupling, and frame.

## 3.1.1 Flywheel Design

The flywheel is the most critical mechanical component. Its moment of inertia, combined with its operating speed, determines the angular momentum and hence the gyroscopic stability of the system.

### 3.1.1.1 Design Requirements

The flywheel design requirements were:

Moment of inertia I >= 0.004 kg.m2 (from stability analysis)

Geometrically symmetric to minimise initial dynamic imbalance

Machinable from locally available mild steel stock on CBU's CNC lathe

Balance achievable to ISO G6.3 grade using the CBU balancing rig

Structural integrity with safety factor >= 2.0 at 10000 RPM (25% overspeed)

Mass <= 1.2 kg to remain within the bearing load capacity calculations

### 3.1.1.2 Geometry

The flywheel was designed as a solid mild steel disk: diameter 215.8 mm, thickness 20 mm. Eight peripheral bolts were added to increase the effective moment of inertia: four M12 bolts at radius 99 mm and four M6 bolts at radius 95 mm. The bolts are indexed at 90-degree spacing (M12) and 45-degree spacing (M6) to maintain geometric symmetry. The hub bore (8 mm) is an interference fit on the shaft; a set screw provides secondary retention. Total design mass including bolts: 0.8606 kg.

[Figure 2.2: Flywheel engineering drawing -- all dimensions in mm]

[Figure 3.1: Mass distribution diagram showing bolt positions and radii used in inertia calculation]

## 3.1.2 Shaft Design

The main shaft is EN3B mild steel, 8 mm diameter, 120 mm long. The shaft connects the motor coupling hub at one end to the flywheel hub at the other, and is supported by two RS608 ball bearings located 35 mm from each end.

The shaft diameter was not sized independently; it was set to 8 mm to match the bore of the RS608 bearings already fixed by the inherited frame geometry (Section 3.1.3), since re-boring the existing bearing housings to accept a larger shaft and bearing pair would have required rebuilding the frame, outside the scope of this improvement project. EN3B mild steel was chosen over an alloy steel (e.g. EN8) because it is readily available from CBU's workshop stock and easier to turn to the bearing-fit tolerance on the available lathes; the structural analysis in Section 3.2.4 later confirmed a combined safety factor of 49.6 at the design load, an order of magnitude beyond what the load case required, so the lower-cost, easier-to-machine material was the appropriate choice.

The bearing spacing of 50 mm was determined by the frame geometry inherited from the previous team. A wider spacing would reduce shaft deflection under load, but the existing bearing housings could not be moved without rebuilding the frame. The 50 mm span was verified adequate in the structural analysis (Section 3.2.4), which found a bending stress of only 1.31 MPa against a safety factor of 328.

[Figure 2.4: Shaft and bearing sub-assembly -- exploded view showing shaft, inner and outer bearing races, and flywheel hub]

## 3.1.3 Bearing Selection

The bearing bore was fixed at 8 mm by the shaft diameter (Section 3.1.2), which narrowed the search to standard 8 mm-bore deep groove ball bearings available through CBU's regional suppliers. RS608 shielded deep groove ball bearings were selected over an open (unshielded) equivalent and a 2RS double rubber-sealed variant: the shielded type is factory-packed with sufficient grease for its rated life without the extra running friction of contact seals, and it was the variant in local stock without an import lead time the project's timeline could not absorb. Key parameters:

| Parameter | Value |
| --- | --- |
| Bore diameter | 8 mm |
| Outer diameter | 22 mm |
| Width | 7 mm |
| Dynamic load rating C | 3450 N |
| Static load rating C0 | 1560 N |
| Max speed (grease lube) | 32000 RPM |
| Operating temperature | -20 to +120 degC |
| Lubrication | Grease -- sealed, no maintenance |

The dynamic load rating was verified adequate, not merely assumed, in the bearing life calculation of Section 3.2.2: at the applied radial load of 4.22 N and an operating speed of 8000 RPM, the calculated L10 life is 2371 hours -- a 98.8x margin over the 24-hour continuous-operation target. This margin reflects standardising on a single stocked, no-maintenance bearing size shared with the shaft-bore constraint above, rather than introducing a second bearing purely to trim an already inexpensive component.

## 3.1.4 Motor and ESC Selection

### 3.1.4.1 Motor Selection

The 2200 KV BLDC motor was chosen based on the speed-voltage relationship n = KV * V, calculated in Section 3.2.1. At the design supply voltage of 4.84 V, the no-load speed is approximately 8000 RPM.

KV was not the only free variable: for a fixed 8000 RPM target, a lower-KV motor needs a higher supply voltage and a higher-KV motor needs a lower one, while the torque constant k_t (available torque per amp) scales inversely with KV. Table 7a compares the selected 2200 KV motor against two commercially available alternatives at the same target speed, using these relationships.

Table 7a: Motor KV Selection -- Trade-off at Fixed 8000 RPM Target

| Motor Option | Voltage for 8000 RPM | Relative k_t | Suitability |
| --- | --- | --- | --- |
| 1400 KV | ~5.71 V | Higher (~1.57x) | Rejected -- exceeds the available 5V-class supply rail, would need an extra boost stage |
| 2200 KV (chosen) | 4.84 V | Baseline (0.00434 N.m/A) | Selected -- matches the available supply and gives adequate stall torque (0.0434 N.m at 10 A) |
| 3200 KV | ~2.50 V | Lower (~0.69x) | Rejected -- impractically low voltage relative to wiring/connector drop; reduced k_t cuts torque margin |

Full electrical parameters for the selected motor:

| Parameter | Value |
| --- | --- |
| KV rating | 2200 RPM/V |
| No-load current I0 | 1.5 A |
| Maximum current | 10 A |
| Stator resistance R | 0.12 ohm |
| Number of poles | 12 (6 pole pairs) |
| Mass | 45 g |
| Shaft diameter | 3 mm |

Table 8: Motor Electrical Parameters

| Parameter | Value |
| --- | --- |
| Back-EMF constant k_e | 0.00434 V.s/rad |
| Torque constant k_t | 0.00434 N.m/A |
| Stall torque | 0.0434 N.m at 10 A |
| Running torque | ~0.013 N.m at 3 A steady-state |
| Electrical power | 48.4 W at full load |
| Mechanical efficiency | 86.6% |

### 3.1.4.2 ESC Design and Selection

The flywheel motor is driven by a commercial, off-the-shelf 30 A BLDC ESC rather than a purpose-built driver stage. A custom three-half-bridge driver (IRFZ44N MOSFETs with IR2101 gate drivers) was modelled in Proteus during the control-system simulation work (Section 3.2.5) because Proteus has no library part for a commercial ESC, but that custom circuit was a simulation-only stand-in -- it was never built into the physical prototype. The physical machine uses the commercial ESC described below throughout.

Key specifications, from the manufacturer datasheet:

| Parameter | Value |
| --- | --- |
| Continuous / peak current | 30 A continuous; 40 A for 10 s |
| Input voltage range | 2-4S LiPo/Li-ion (7.4-14.8 V) or 5-12 cell NiMH/NiCd |
| BEC output | 5 V, 3 A |
| Max electrical speed (12-pole motor) | 35000 RPM |
| Control signal | Standard 50-60 Hz PWM, 1-2 ms pulse width |
| Protections | Low-voltage cut-off, over-heat cut-back above 110C, throttle-signal-loss cut-back, abnormal-start cut-off |
| Mass / size | 32 g / 55 x 26 x 13 mm |

Table 8a: Commercial ESC Specifications

This ESC was selected over building a custom driver for the physical prototype for four reasons. First, its 30 A continuous / 40 A surge rating is far beyond the motor's own 10 A maximum current draw and ~3 A steady-state running current, so it operates well inside its own thermal and electrical limits under every load case this project produces -- a hand-built driver would need to be engineered and verified to reach the same margin for no operational benefit. Second, its built-in protections (over-temperature cut-back, low-voltage cut-off, throttle-signal-loss cut-back) directly satisfy the safety requirements the project already needed, without adding them in firmware or extra hardware. Third, it accepts the same standard 50-60 Hz, 1-2 ms PWM signal already generated by the ESP32 LEDC peripheral for the PID output stage (Section 3.2.5), so it interfaces directly with no additional signal conditioning. Fourth, as a pre-tested commercial unit it removed a point of failure risk from the build schedule that a hand-built MOSFET/gate-driver stage would have carried, which is why the custom design was kept to a simulation-only role instead.

Because a commercial ESC drives the motor by pulse-width-modulating the raw supply rail rather than physically stepping the supply voltage down, the 4.84 V figure used in the speed calculation of Section 3.2.1 is the effective average voltage delivered to the motor windings at the operating throttle command, not the ESC's raw input voltage. The physical prototype's raw supply rail is a bench AC-DC adapter (110-240 VAC input; selectable 12/15/16/18/19 V at 4.5 A or 20/24 V at 5 A DC output) set to its 12 V tap, chosen because it falls within the ESC's rated 2-4S LiPo / NiMH-NiCd input window and was available on hand in place of a dedicated battery pack. At a 12 V supply rail, the ESP32's PWM throttle command was tuned to a duty cycle of approximately 40% (4.84 V / 12 V) to produce the 8000 RPM no-load operating speed assumed throughout the design calculations. If a different supply voltage is used, this duty cycle must be re-tuned to keep the no-load speed -- and therefore the stability, inertia, and power calculations of Section 3.2.1 -- valid.

Verification of the throttle interface and current draw against the datasheet's ratings is reported experimentally in Section 5.2 (Test 1: Motor Control and ESC Characterisation).

## 3.1.5 Coupling Design

A rubber spider (jaw) coupling connects the motor shaft to the flywheel shaft, replacing the rigid, in-line coupling used in the previous prototype. Three alternatives were considered against the requirement to isolate motor commutation vibration from the precision-balanced flywheel shaft while tolerating the small misalignment inevitable in a hand-assembled frame:

Rigid (sleeve/set-screw) coupling -- the previous design. Rejected: transmits vibration and shock directly into the flywheel shaft and bearings, and has zero misalignment tolerance.

Bellows coupling -- precise, near-zero backlash. Rejected: designed for torsional stiffness and positioning accuracy, not vibration damping, so it would not address the commutation-vibration problem.

Elastomeric spider (jaw) coupling -- chosen. The elastomeric insert absorbs shock during motor engagement with the already-spinning flywheel, isolates commutation vibration from reaching the flywheel shaft and bearings, and tolerates up to 1 degree angular and 0.2 mm parallel misalignment.

[Figure 2.5: Spider coupling -- exploded view showing motor hub, elastomeric insert, and flywheel hub]

## 3.1.6 Frame Design

The steel frame was inherited from the previous team and modified. Modifications: bearing housings redrilled and reamed to accept RS608 bearings; motor mounting plate reinforced with 25 x 3 mm flat bar bracing; all surfaces wire-brushed and repainted.

[Figure 2.3: Internal component layout -- side section view showing frame, bearings, shaft, motor, and coupling positions]

# 3.2 Component Analysis

This section verifies the components selected in Section 3.1 through engineering calculation, and presents the control system, software, and simulation work that underpins the design.

## 3.2.1 Design Calculations

### 3.2.1.1 Moment of Inertia -- Flywheel Disk

For a uniform solid disk (Hibbeler, 2015, p. 388):

I_disk = (1/2) * m * r2

I_disk = (1/2) * 0.8606 * (0.1079)2

I_disk = (1/2) * 0.8606 * 0.011642

I_disk = 0.5 * 0.010022 = 0.005011 kg.m2  (recorded as 0.005006 allowing for rounding)

### 3.2.1.2 Additional Inertia from Rim Bolts

Each bolt is a point mass at its bolt-circle radius:

I_M12 = 4 * m_M12 * r_M122 = 4 * 0.035 * (0.099)2

I_M12 = 4 * 0.035 * 0.009801 = 0.001372 kg.m2

I_M6 = 4 * m_M6 * r_M62 = 4 * 0.008 * (0.095)2

I_M6 = 4 * 0.008 * 0.009025 = 0.000289 kg.m2

I_bolts = I_M12 + I_M6 = 0.001372 + 0.000289 = 0.001661 kg.m2

I_total = 0.005006 + 0.001661 = 0.006667 kg.m2

### 3.2.1.3 Operating Angular Velocity

omega_s = N * (2*pi / 60) = 8000 * 0.10472 = 837.76 rad/s

### 3.2.1.4 Angular Momentum

L = I * omega_s = 0.005006 * 837.76 = 4.194 kg.m2/s   (Hibbeler, 2015)

### 3.2.1.5 Gyroscopic Torque at 0.5 rad/s Precession

T_g = I * omega_s * omega_p = 0.005006 * 837.76 * 0.5 = 2.097 N.m   (Meriam et al., 2016)

### 3.2.1.6 Gravitational Overturning Torque at 5 degrees Tilt

Parameters: m = 0.8606 kg, g = 9.81 m/s2, h = 0.05 m, theta = 5 deg:

T_grav = m * g * h * sin(theta)

T_grav = 0.8606 * 9.81 * 0.05 * sin(5 deg)

T_grav = 0.422 * 0.08716 = 0.0368 N.m

### 3.2.1.7 Stability Criterion Check

T_g / T_grav = 2.097 / 0.0368 = 56.98 ~ 57   (Ogata, 2010)

The gyroscopic torque exceeds the gravitational overturning torque by a factor of 57. The stability criterion is satisfied with a very large margin. The minimum stable speed (where margin factor = 1.0) is approximately 8000 / 57^0.5 = 8000 / 7.55 = 1060 RPM -- well below any credible operating condition.

### 3.2.1.8 Motor Back-EMF Constant

k_e = 1 / (KV * (2*pi/60)) = 1 / (2200 * 0.10472) = 0.00434 V.s/rad

### 3.2.1.9 Required Supply Voltage

V_back_emf = k_e * omega_s = 0.00434 * 837.76 = 3.636 V

V_resistive = I_running * R = 10.0 * 0.12 = 1.200 V

V_supply = V_back_emf + V_resistive = 3.636 + 1.200 = 4.836 V ~ 4.84 V

### 3.2.1.10 Stored Kinetic Energy

E_k = (1/2) * I * omega2 = (1/2) * 0.005006 * (837.76)2

E_k = 0.002503 * 701602 = 1756.7 J = 0.488 Wh

### 3.2.1.11 Motor Power

P_elec = V * I_max = 4.84 * 10.0 = 48.4 W

P_mech = T_running * omega_s = 0.05 * 837.76 = 41.9 W

Efficiency = P_mech / P_elec = 41.9 / 48.4 = 86.6%

### 3.2.1.12 Flywheel Hoop Stress at Overspeed (10000 RPM)

The hoop stress in a rotating disk at radius r from the centre is (Meriam, Kraige and Bolton, 2016):

sigma_hoop = (3+nu)/8 * rho * omega2 * (ro2 + ri2)

Using Poisson's ratio nu = 0.3, density rho = 7850 kg/m3, ro = 0.1079 m, ri = 0 (solid disk), omega = 10000 * 2pi/60 = 1047.2 rad/s:

sigma_hoop = (3.3/8) * 7850 * (1047.2)2 * (0.1079)2

sigma_hoop = 0.4125 * 7850 * 1096640 * 0.01164

sigma_hoop = 0.4125 * 7850 * 12765 = 41.3 MPa

The mild steel tensile strength is 400 MPa. Safety factor at overspeed = 400 / 41.3 = 9.7. The flywheel is extremely safe against structural failure even at 25% above the design speed.

Table 7: Summary of All Design Calculations

| Calculation | Formula | Result | Target |
| --- | --- | --- | --- |
| Disk moment of inertia | I=(1/2)mr2 | 0.005006 kg.m2 | >=0.004 |
| Bolt inertia | I=sum(m_i*r_i2) | 0.00166 kg.m2 | -- |
| Total inertia | I_disk+I_bolts | 0.00667 kg.m2 | -- |
| Angular velocity | omega=N*2pi/60 | 837.76 rad/s | -- |
| Angular momentum | L=I*omega | 4.194 kg.m2/s | -- |
| Gyroscopic torque | T=I*omega_s*omega_p | 2.097 N.m | -- |
| Gravity torque (5 deg) | T=mgh*sin(theta) | 0.0368 N.m | -- |
| Stability factor | T_g/T_grav | 57 | >=1 |
| Supply voltage | V=k_e*omega+I*R | 4.84 V | -- |
| Kinetic energy | E=(1/2)*I*omega2 | 1756.7 J | -- |
| Motor efficiency | P_mech/P_elec | 86.6% | -- |
| Hoop stress (overspeed) | sigma=(3+nu)/8*rho*omega2*ro2 | 41.3 MPa | Safety factor 9.7 |

## 3.2.2 Bearing Life Calculation

Bearing life was calculated using the ISO 281 L10 formula for ball bearings (Meriam, Kraige and Bolton, 2016):

L10 = (C/P)3 * (10^6 / (60*N))   [hours]

Determining the equivalent dynamic load P:

P_radial = (m_flywheel * g) / 2 = (0.8606 * 9.81) / 2 = 4.22 N

After dynamic balancing to ISO G6.3, residual imbalance adds at most 0.004 N. Taking P = 4.22 N (radial only; no significant axial load in this application):

L10 = (3450 / 4.22)3 * (10^6 / (60 * 8000))

L10 = (817.5)3 * 2.0833

L10 = 5.462 x 10^8 * 2.0833 = 1.138 x 10^9 revolutions

L10 = 1.138 x 10^9 / (8000 * 60) = 2371 hours

The L10 life of 2371 hours at operating speed is 98.8 times longer than the 24-hour endurance target. Bearing failure is not a credible failure mode for the test duration.

Table 9: Bearing Selection Parameters and Calculated Life

| Parameter | Value | Notes |
| --- | --- | --- |
| Bearing model | RS608 | Deep groove ball bearing |
| Bore / OD | 8 mm / 22 mm |  |
| Dynamic rating C | 3450 N | From data sheet |
| Operating speed | 8000 RPM | Nominal |
| Applied load P | 4.22 N | Half flywheel weight |
| L10 life | 2371 hours | ISO 281 formula |
| Margin over target | 98.8x | Target 24 hours |

## 3.2.3 Thermal Analysis

A steady-state first-order thermal model estimates motor winding temperature rise during continuous operation. At steady-state (constant speed), the running current is approximately 3 A. Copper winding losses:

P_copper = I2 * R = (3.0)2 * 0.12 = 1.08 W

Motor casing surface area (cylinder approximation):

A_casing = pi * D * L = pi * 0.025 * 0.030 = 0.00236 m2

Steady-state thermal balance (heat in = heat out):

P_copper = h_conv * A_casing * delta_T

delta_T = P_copper / (h_conv * A_casing) = 1.08 / (10 * 0.00236) = 45.8 degC

With maximum ambient of 30 degC, predicted motor surface temperature is 30 + 45.8 = 75.8 degC. Winding temperature (typically 10-20 degC higher than surface) is approximately 86-96 degC. BLDC motors are typically rated to >= 100 degC winding temperature -- the margin is small but acceptable, and the dashboard temperature alert provides further protection.

Table 10: Thermal Analysis Inputs and Results

| Parameter | Value | Notes |
| --- | --- | --- |
| Steady-state current | 3.0 A | Measured during constant-speed run |
| Stator resistance R | 0.12 ohm | From motor data sheet |
| Copper losses | 1.08 W | I2R at steady-state current |
| Casing surface area | 0.00236 m2 | Cylinder approximation |
| Conv. coefficient h | 10 W/m2.K | Natural convection, conservative |
| Predicted delta_T | 45.8 degC | At steady state |
| Max surface temp | 75.8 degC | At 30 degC ambient |
| Motor winding limit | >100 degC | Typical BLDC limit |
| Safety margin | ~14 degC | Adequate with monitoring |

## 3.2.4 Structural Analysis of Main Shaft

The shaft is subjected to a bending moment from the radial load at the flywheel position between the two bearing supports. After dynamic balancing, the dominant load is the flywheel weight: 0.8606 * 9.81 = 8.44 N applied at the shaft centre. With a bearing span of 50 mm and the flywheel at the mid-span:

M_max = F * L / 4 = 8.44 * 0.050 / 4 = 0.1055 N.m

Bending stress in the 8 mm diameter shaft:

sigma_b = M_max * (d/2) / (pi * d4 / 64)

sigma_b = 0.1055 * 0.004 / (pi * (0.008)4 / 64)

sigma_b = 0.000422 / (3.217 x 10^-10) = 1.31 MPa

The EN3B mild steel has a tensile strength of 430 MPa. The shaft safety factor against static bending failure is 430 / 1.31 = 328 -- the shaft is completely adequate. Adding the torsional stress from motor torque (0.05 N.m, yielding tau = 4.97 MPa via J = pi*d4/32) and combining with von Mises criterion:

sigma_vm = sqrt(sigma_b2 + 3*tau2) = sqrt(1.312 + 3*4.972) = 8.67 MPa

Safety factor (von Mises) = 430 / 8.67 = 49.6

The shaft is very safe. Even with 10x the torque, the safety factor would remain above 4.0.

Table 11: Shaft Structural Analysis Results

| Load Type | Stress | Safety Factor |
| --- | --- | --- |
| Bending | 1.31 MPa | 328 |
| Torsion | 4.97 MPa | 86.5 |
| Combined (von Mises) | 8.67 MPa | 49.6 |

[Figure 3.4: Shaft bending moment diagram under flywheel weight load]

## 3.2.5 Control System Design

### 3.2.5.1 Plant Model

The motor-flywheel plant was modelled as a second-order system. The motor electrical model relates input voltage V to current I:

V = k_e * omega + I * R      (inductance neglected)

The mechanical model relates motor torque to flywheel acceleration:

T_motor - T_friction = I_flywheel * (d_omega/dt)

T_friction at running speed is approximately 0.01 N.m, estimated from the no-load current of 1.5 A via T_friction = k_t * I0 = 0.00434 * 1.5 = 0.0065 N.m (rounded to 0.01 N.m). The combined electrical-mechanical transfer function from PWM duty cycle to angular velocity was constructed in Simulink and used by the PID Tuner.

### 3.2.5.2 PID Implementation on ESP32

The discrete-time PID implementation uses the backward Euler method for the integral term and a first-order derivative filter to limit noise amplification. The integral term is clamped (anti-windup) whenever the controller output saturates, preventing the integral from accumulating during the motor ramp-up phase.

| PID Parameter | Value |
| --- | --- |
| Kp | 0.85 |
| Ki | 2.40 |
| Kd | 0.015 |
| Derivative filter N | 100 |
| Update period | 20 ms (50 Hz) |
| Anti-windup | Integral clamping -- saturation detection |
| Output min/max | 1050 / 2000 microseconds (PWM pulse width) |

### 3.2.5.3 Simulation Results -- PID Tuner Output

MATLAB Simulink PID Tuner was set to target 2 seconds settling time and 45 degrees phase margin. The initial gains from the auto-tuner were Kp = 0.79, Ki = 2.21, Kd = 0.013. These were tested in Simulink for five disturbance scenarios and adjusted slightly (by less than 10%) by manual iteration before hardware deployment.

## 3.2.6 Software and Monitoring System Architecture

### 3.2.6.1 ESP32 Firmware Architecture

The firmware main loop (20 ms period) performs these tasks in sequence:

1. Read MPU6050 via I2C (6-axis data: ax, ay, az, gx, gy, gz)

2. Update Kalman filter -- compute filtered tilt angles for X and Y axes

3. Accumulate 64 accelerometer Z samples at 500 Hz for FFT vibration analysis

4. When 64 samples complete: run arduinoFFT to find dominant frequency; convert to RPM estimate

5. Run PID control law on RPM error; compute ESC PWM pulse width

6. Output PWM signal to ESC via timer peripheral

7. Every 5th cycle (100 ms): send JSON telemetry over serial USB

8. Every 5th cycle (100 ms): send JSON telemetry over Wi-Fi TCP if connected

9. Check for incoming commands on serial and Wi-Fi channels

### 3.2.6.2 Node.js Server Architecture

The Express server handles the following concurrently:

1. Serial port reader: parses JSON telemetry from ESP32 at 115200 baud

2. Wi-Fi TCP server: listens on port 5001 for ESP32 Wi-Fi connections

3. Safety monitor: on each reading, compares values against configured thresholds; triggers emergency stop if any critical threshold is exceeded

4. WebSocket server: broadcasts readings and alerts to all connected dashboards

5. PostgreSQL writer: inserts readings, sessions, and alert records via Drizzle ORM

6. REST API: session management, motor control commands, settings get/set, historical data export

### 3.2.6.3 React Dashboard Architecture

The React dashboard is a single-page application built with Vite, Tailwind CSS, shadcn/ui components, and Recharts for data visualisation. Key UI sections:

Live charts: 60-second rolling window for all six sensor channels, updated at 4 Hz via WebSocket

Stat cards: current values with trend arrows for RPM, tilt X/Y, temperature, vibration, PWM

System events panel: scrollable alert history with severity colour coding

Motor control panel: session start/stop, motor on/off, emergency stop button

Settings page: configurable thresholds, connectivity settings, dark/light toggle

Connect dialog: step-by-step Wi-Fi and USB serial connection instructions

[Figure 5.2: Monitoring dashboard -- live RPM chart, stat cards, motor control panel, and session information]

[Figure 5.3: Dashboard -- system events panel showing alert history with severity colour coding]

## 3.2.7 MATLAB Simulink Simulation

Two simulations were conducted to validate the control system design:

### 3.2.7.1 Uncontrolled System Simulation

With PID disabled, the flywheel was initialised at 8000 RPM and a 5-degree tilt disturbance applied at t = 5 s. Under friction alone, the speed decayed to 5000 RPM after approximately 8 minutes of simulated time. Without speed regulation, the gyroscope would eventually fall below the stability limit, confirming that active control is essential for the 24-hour target.

### 3.2.7.2 Controlled System Simulation -- Step Response

With PID active (gains from Simulink Auto-Tuner), the same 5-degree disturbance at t = 5 s produced a speed dip of approximately 420 RPM (5.25%), followed by recovery to within +/-50 RPM of the 8000 RPM setpoint in 1.8 seconds. Overshoot was 8.5%. Both metrics met the design targets (<2 second settling, <10% overshoot).

### 3.2.7.3 24-Hour Continuous Simulation

A 24-hour simulation with step disturbances applied every 2 hours showed that the PID controller maintained speed within +/-80 RPM of setpoint throughout. Motor temperature (from the thermal model) reached steady state within 30 minutes and remained stable. This provided confidence in the design before the physical endurance test.

[Figure 4.0: MATLAB Simulink -- complete system block diagram with all four subsystems and feedback connections labelled]

[Figure 4.1: Simulink PID controller subsystem showing K_p, K_i, K_d blocks, integrator clamping, and output saturation limits]

[Figure 4.2: Uncontrolled vs. controlled simulation response comparison -- speed vs. time following 5-degree disturbance]

[Figure 4.3: PID step response with rise time, settling time, and peak overshoot annotated]

[Figure 4.4: Simulink motor electrical model subsystem]

## 3.2.8 Circuit Design and Simulation

The circuit was fully designed in Proteus before any components were purchased or soldered. The design consists of five functional blocks:

1. Power Regulation: 12 V from bench supply regulated to 5 V by LM7805 (for logic) and 3.3 V by AMS1117-3.3 (for ESP32). Each regulator has 100 uF and 100 nF ceramic decoupling capacitors at both input and output pins.

2. ESP32 Controller: Generates 50 Hz PWM via the ESP32 LEDC (LED Control) peripheral, which provides higher accuracy and lower jitter than software-timed output. I2C on GPIO21/22 for MPU6050. UART0 for serial-USB bridge. Antenna for Wi-Fi TCP connection.

3. MPU6050 IMU: 3.3 V supply with 100 nF decoupling. I2C pull-up resistors (4.7 kohm) to 3.3 V. FSYNC and INT pins connected per InvenSense application note (InvenSense, 2013, p. 31).

4. Custom ESC Module: Three half-bridge circuits using IRFZ44N N-channel power MOSFETs. IR2101 gate driver ICs provide 100 mA peak gate current and handle the high-side bootstrap charge. 0.01 ohm shunt resistors on each low-side for current sensing. 100 uF + 100 nF decoupling on the motor supply bus.

5. Protection Circuits: TVS diode (1.5KE33A) bidirectional across motor supply rails to clamp inductive kickback from motor commutation. Fuse (3 A fast-blow) in series with motor supply positive rail.

Proteus simulation confirmed: correct gate drive timing for all six MOSFETs; adequate bootstrap charge time at operating frequency; correct back-EMF sensing waveforms; and absence of noise coupling from the motor drive traces to the sensor supply lines.

[Figure 5.0: Complete circuit schematic in Proteus (2026) -- power supply, ESP32, MPU6050, and ESC sections]

[Figure 5.1: Proteus simulation waveforms -- ESC gate drive signals for all six MOSFETs, showing dead-time and bootstrap charge intervals]